M A T E R I A L   S A F E T Y   D A T A   S H E E T

TEXAS REFINERY CORP.

840 N. Main Street

Fort Worth, Texas  76164

(817) 332-1161

24 Hr. Emergency Telephone No.:   CHEMTREC 800-424-9300

Preparation/Revision Date - June, 2007

Supersedes - May, 2005

****** I.  PRODUCT IDENTIFICATION ******

TRADE NAME - SUPER-EEZ Penetrating Oil (Aerosol)

CHEMICAL FAMILY - Hydrocarbon                          PRODUCT CODE:  6555-001

****** II.  HAZARDOUS INGREDIENTS ******

l.  Stoddard Solvent

CAS# 8052-41-3         PEL: 500 ppm          TLV: 100 ppm         WT.% 30-45

2.  Heavy Paraffinic Distillates (petroleum) (oil mist)

CAS# 64742-54-7        PEL: 5mg/m3            TLV: 3mg/m3          WT.% 30-45

3.  Carbon Dioxide (propellant)

CAS# 124-38-9          PEL: 9000mg/m3         TLV: 9000mg/m3       WT.% 20-35

This material is not known to contain any chemicals on the SARA Section 313 list at a concentration greater than 1.0% or any carcinogenic chemical at a concentration greater than 0.1%.

****** III.  PHYSICAL/CHEMICAL DATA ******

(Liquid only w/o propellant)

BOILING POINT (OF/C):  318/158                 SPECIFIC GRAVITY (H20=1):  0.82

VAPOR PRESSURE (mm Hg): < 2.0 @ 70oF          PERCENT VOLATILE BY VOL.:  47

VAPOR DENSITY (Air=1): Not established        SOLUBILITY IN WATER:  Negligible

EVAPORATION RATE (Butyl Acetate=1):  Not established

APPEARANCE AND ODOR - Amber liquid; aromatic solvent odor

****** IV.  FIRE AND EXPLOSION HAZARD DATA ******

FLASH POINT (OF/C):  145/63 min. (PMCC) (liquid without the propellant)

FLAMMABLE LIMITS:  Not established

EXTINGUISHING MEDIA:  Dry Chemical, CO2, Water Fog

SPECIAL FIRE FIGHTING PROCEDURES:  Use air-supplied breathing equipment.  Cool

exposed containers with water spray.

UNUSUAL FIRE AND EXPLOSION HAZARDS:  Aerosol cans at temperatures above 120oF

(49oC) may explode.  Treat as cylinders of compressed gas.

NFPA:          HEALTH:  1        FLAMMABILITY:  2        REACTIVITY:  0

HMIS:          HEALTH:  1        FLAMMABILITY:  2        REACTIVITY:  0   PPE:  B

****** V.  HEALTH HAZARD DATA ******

TLV/PEL:  Not established - See Section II

CARCINOGENS - None

PRINCIPAL ROUTES OF EXPOSURE - Inhalation, Skin and Eyes

EFFECTS OF OVEREXPOSURE - Inhalation of high vapor concentrations is irritating to the respiratory system; may have results ranging from dizziness and headaches to unconsciousness.  May have other central nervous system effects.  May dry and de-fat the skin leading to irritation and dermatitis.  Will cause irritation to eyes.  Ingestion of product is considered to be a low order of acute oral toxicity, but minute amounts aspirated into the lungs during ingestion may cause mild to severe pulmonary injury and possibly death.

EMERGENCY AND FIRST AID PROCEDURES - Remove to fresh air if inhaled.  Administer

oxygen if breathing is irregular or artificial respiration if breathing has stopped.  Remove contaminated clothing and wash skin with soap and water.  Flush eyes with water for 15 minutes; obtain medical attention if irritation persists.  If ingested, DO NOT induce vomiting.  Vomiting may lead to pneumonitis, which could be fatal.  Get medical attention immediately.

MEDICAL CONDITIONS GENERALLY AGGRAVATED BY EXPOSURE - Unknown

NOTES TO PHYSICIAN - None

****** VI.  REACTIVITY DATA ******

STABILITY - Stable

CONDITIONS TO AVOID - Avoid open flames, hot glowing surfaces or electric arcs.

INCOMPATABILITY (Materials to Avoid) - Strong oxidizers and strong acids.

HAZARDOUS DECOMPOSITION PRODUCTS – Oxides of carbon from burning.

HAZARDOUS POLYMERIZATION - Will not occur

****** VII.  PRECAUTIONS FOR SAFE HANDLING AND USE ******

STEPS TO BE TAKEN IN CASE MATERIAL IS RELEASED OR SPILLED - Small spills: recover

free product.  Add absorbent to spill area; pick up and place in waste container.

WASTE DISPOSAL METHOD - Conform to Federal, State and/or Local disposal regulations.  Do not puncture or incinerate the aerosol containers.  When all liquid is expelled, continue to depress button until gas has been exhausted.

PRECAUTIONS TO BE TAKEN IN HANDLING AND STORAGE - Keep away from open flames, hot

surfaces or electric arcs.  Do not expose to temperatures above 120oF (49oC) due to contents being under pressure.

OTHER PRECAUTIONS - Avoid smoking in presence of vapors.

****** VIII.  CONTROL MEASURES ******

RESPIRATORY PROTECTION - None required with adequate ventilation.  Where vapor

concentrations exceed the TLV, respirator with organic canister should be used.

VENTILATION - Maintain concentrations below TLV of ingredients in Section II

PROTECTIVE GLOVES - Neoprene, viton or polyvinyl coated

EYE PROTECTION - Safety glasses with side shields or goggles

OTHER PROTECTIVE EQUIPMENT - None

WORK/HYGIENIC PRACTICES - Observe good personal hygiene practice when handling this material.

****** IX.  TRANSPORTATION DATA ******

DOT HAZARD CLASS - ORM-D                             		 IDENTIFICATION NUMBER - N/A

IMCO DESCRIPTION - Aerosols (Maximum 1 l.),Limited Quantity, IMCO CLASS - 2

UN NUMBER - 1950                                     		 LABEL - Flammable Gas

FOR ADDITIONAL INFORMATION CONTACT:  Regulatory Affairs Office

(800) 827-0711   Ext. 277

THIS INFORMATION IS BEING SUPPLIED TO YOU UNDER OSHA "RIGHT TO KNOW" REGULATION 29 CFR 1910.1200 AND IS OFFERED IN GOOD FAITH.  THE INFORMATION CONTAINED HEREIN IS BASED ON THE DATA AVAILABLE TO US AND IS BELIEVED TO BE TRUE AND ACCURATE.  NO WARRANTY, EXPRESSED OR IMPLIED, REGARDING THE ACCURACY OF THIS DATA, THE HAZARDS CONNECTED WITH THE USE OF THE MATERIAL, OR THE RESULTS TO BE OBTAINED FROM THE USE THEREOF, IS MADE.  TEXAS REFINERY CORP. ASSUMES NO RESPONSIBILITY FOR DAMAGE OR INJURY FROM THE USE OF THE PRODUCT DESCRIBED HEREIN.